Programming Assignments

Hopefully by now you’ve looked over the tutorial and you know basically how python works.  If you didn’t do this, and don’t know how python works at all, then these assignments might be difficult. But who knows? Maybe you’re born with it. Or maybe it’s Maybelline.

So basically, these assignments will consist of you writing code in python that will be altered to be used in lab.  In lab, we’ll modify our code to work on the raspberry Pi, but for now, our only output and input are the console, so we’ll just pretend that we’re using the Raspberry Pi.

These assignments always start on a fresh page, and will include information about the purpose of the assignment and special information required to complete it.

Once the program is finished, it should react predictably, and you may compare your program to a sample of the desired output.  As long as they act similarly, then you should be fine to move on.

The creation of these programs are useful exercises, but they aren’t the end goal.  Once you arrive at the Raspberry Pi lab, you will be able to convert these exercises so they run and perform specific tasks on your microcontrollers.  This process is detailed at the end of 00example.

So start off by looking through 00example to get an idea of what’s expected of you.  The files associated with the assignment (expected output, base code to start from etc.) are housed in the assignments folder in the same directory as this file.

Assignment 00Example

This assignment is related to turning an LED on and off.  The way to do that (once it’s hooked up to your circuit) is by toggling the I/O (input output) port that it’s connected to.  So we’re going to pretend that the variable ‘a’ controls the LED, and that we can turn it on or off by setting it true or false.  This lab is easier with the sleep command from the time library.

The partial code will be in an attached .py file, and will it look something like this:

import time

a = False

# write code to change and print the value of a every second
# in a infinite loop (remember that ctrl-c or command-c
# is the key combination used to escape from an infinite loop.

Continued on next page

There will also be an example expected output code that will help you design your code.  For this assignment, it may look something like this, and it’s usually stored in just a pdf.

Python 3.6.0 (v3.6.0:41df79263a11, Dec 23 2016, 07:18:10) [MSC v.1900 32 bit (Intel)] on win32

Type "copyright", "credits" or "license()" for more information.

>>>

= RESTART: C:/Users/David/Documents/TE 205 Python/Assignments/00 Example.py =

True

False

True

False

Traceback (most recent call last):

File "C:/Users/David/Documents/TE 205 Python/Assignments/00 Example.py", line 6, in <module>

time.sleep(1)

KeyboardInterrupt

>>>

Continued on next page

Here’s an example solution to the previous problem.  If you had trouble understanding what was expected and/or how to write these programs, don’t worry.  This is the first time these materials have been utilized, and there’s plenty of help available.

import time

a = False

# write code to change and print the value of a every second
# in a while loop that is always true (do you remember that ctrl-c
# breaks you out of an infinite loop)?

while True:

print(a)			#the print lines will have to be changed

sleep(1)

a = not a

For now, don’t worry about this next step, but in lab, we’ll transform the code above into something like the code below

import time

import RPi.GPIO as GPIO

GPIO.setmode(GPIO.BCM) # determines how we refer to pins

GPIO.setup(3, GPIO.OUT) # sets pin 3 to output

a = False

while True:

GPIO.output(3, a) 	# turns on pin 3 if a is true

sleep(1)			#delays for one second

a = not a			#changes the value of a to its negative

Assignment 01onOff

We’ve learned how to turn a variable on and off on repeat, but it’s time to learn how to control it by keyboard input.  In this assignment, your goal is to set the variable ‘a’ to a specific setting by keyboard prompts.  If the user types on, it should turn set a to true and print it (turn the LED on), while if they type off, it should do the opposite.

You should have recognized from the previous lines that we’re going to be using input() quite a bit to prompt the user for the next action quite often.  This prompt is going to be interpreted after it’s submitted, and then the status of the LED should be printed.

The skeleton file for this is 01OnOff.py and a sample output can be found called expected.

Assignment 02fade

So you’ve seen how to turn on and off an LED, now it’s time to learn how to control the exact brightness of it.  As you will learn in the lab, LEDs only have two settings (on/off).  As such, it’s quite difficult to get them to truly dim.  But if they flicker quickly, then they may appear dim or bright to the human eye.  We’ll use something called Pulse Width Modulation (PWM) to accomplish this task.  PWM is accomplished by use of a class that controls a few properties of the PWM like frequency, pin, and duty cycle.  I wrote a class to imitate the one you’ll be using on the actual Raspberry Pi, it’s at the top of the skeleton, and it’s big and gross and disgusting, but avoid modifying it anyways or your code might not convert particularly well.

To make an LED dim or brighten, we need to set the duty cycle; however, to some degree we want to learn how to automate this task.  I want to be able to control this LED from the keyboard so I can set it to a specific brightness, and I want to be able to set it to fade to dim, or shine to bright with the keywords fade or shine.  So there are three commands: numbers change the setting on the LED to a different duty cycle, fade makes the LED dim to off, and shine makes it brighten until it’s at full PWM (there’s also an exit command that makes it quit the program!).

The skeleton code for this project is contained in 02fade.py.  It also imports a virtual PWM class that I created for you to practice with, so virtual.py must be in the same folder as your code when you run it.  An explanation for how virtual.py works is at the top of the basic code.  Whenever you use the commands associated with virtual, it will act (hopefully) as explained in the beginning of the given code.  Example output is contained in the word file expected.

Things to remember:

- int(string) converts a string to a integer so it can be used for changeDutyCycle

- look up the range function if you’re not sure what it does (pay attention to the step variable)

Assignment 03pulse

At this point, you’ve gone over analyzing input and controlling the PWM leds, so let’s go ahead and write one more function, pulse, before combining them.  Pulse will continuously brighten and dim the LED, so the only input from the user should be the command pulse, and then your program should be fine to run.

You should know everything that you need to for this assignment from the previous, assignments, so feel free to reference them as you make this one.  The skeleton for this code (very skimpy this time) is 03pulse.py.  Like the last assignment it relies on the code in the python file virtual to imitate a PWM, so you’ll need the virtual.py in the same folder when it is run.  The expected output is put in a word doc as per usual, and DON’T FORGET THAT CTRL-C (or cmd-c on macs) IS USED TO BREAK LOOPS IN PYTHON!!!!

Assignment 04LED

Okay guys, you’ve gone and finished all of the previous assignments, now it’s time to combine them into one big granddaddy of an assignment!  In this assignment, we’re going to create a program that can accurately change its duty cycle, and can blink and fade and shine and pulse and all of those commands.  This would be a remarkably large main assignment, so it might make sense to pass certain parameters to smaller functions that do most of the actual work.

So I’ve included blink already written into the assignment (completely finished!), and you guys should be able to write the rest on your own because you’re all pretty dang smart (sorry but we have high expectations once you make it to college).  Still if you have any questions, feel free to ask Dr. Jasper or myself for help!

The skeleton file is 04LED.py, and once again, it relies on the virtual.py, so don’t run it if virtual.py isn’t in the same folder.  An expected output isn’t included for this assignment, because the expectations are the same as for the other assignments (and because I didn’t want to make one). Good luck!

and DON’T FORGET THAT CTRL-C (or cmd-c on macs) IS USED TO BREAK LOOPS IN PYTHON!!!!

Tldr:  Blink is already written, take all your previous programs, write them into the program LED as functions, and get your program to run all of them at once.

Assignment 05colors

Okay, that last assignment was about as gross as they get.  We’re gonna take a step back now on this next assignment.  We’re moving on from controlling a single led, to controlling three of them!  In lab, we’ll actually be using Tricolor LEDs, which contain three separate leds in a single unit/package.

The goal of this code is to be able to control separate colors individually, and to transition between them with a trigger (we’ll say if a “1” is entered it transitions).  We’ll have to introduce arrays to get this to work, and an index to move through the colors and track which one is on.

There is a skeleton again, but because we’re using a tricolor LED, virtual doesn’t do a great job of describing the situation, so we’re using virtualTri instead.  You’ll see in the skeleton code how this works.

NOTE:  virtual Tri acts as if the red leg is plugged into pin 3, the green leg is plugged into pin 5, and the blue leg is plugged into pin 7.  If you change these in your code then interesting things may occur

Assignment 06rainbow!

color spectrum for RGB.  Source- wikipedia.org RGB color model

Ordinarily, before attempting 06rainbow! we would do the next assignment, 07combo, but 07combo is more difficult and I like rainbow! better anyways.  In this assignment, we need to create a function that transitions seamlessly through the different colors that our tricolor LED can generate.  For example see this graph that I stole- I mean- properly sourced- from Wikipedia!

So that’s what we want to make!  In order to do this, we need to increase and decrease the red green and blue pins such that each peaks at unique times!  There are many different ways to accomplish this; however, I would like you to do it by creating a system that imitates the following chart when prompted.  So all colors begin off, and then red rises to a peak.  As red falls, green rises until it peaks and then the same happens to green and blue.  You may notice that the beginning of the cycle is different from the other sections, so this may need to be programmed separately from the rest.

For the repeating cycle, you may notice that there is always an increasing color that goes to 100 and another color that is decreasing to 0, and these two colors are always in the same order.  Hint: you may be able to write a generic code to increase one item in a list of colors while decreasing another and then cycle through when one of them reaches a maximum.  In addition, if the rising one is 75, then the falling one is 25… there might be a pattern to exploit here.

Assignment 07pick

This assignment introduces a slightly more difficult challenge for us: we need to be able to select the exact color that we would like through either a series of command prompts or an individual command that incorporates all sections.  So there are only two goals of this assignment:  generate a system of picking an exact color which might be a mix of the three primary additive colors, red green and blue (not the three primary colors used in painting), and make sure that that system works with your previous code for rainbow and colors.

There is a skeleton provided for this assignment, 07pick.py that includes some of the libraries and headers you may need loaded.  You should also load code from your previous assignments to make things easier.

Hint: how would you want to input code?  Personally, I would want to be prompted for hex codes, or asked for each color value individually.  Entering a nine digit number might be unwieldy.

Assignment 08TriLED

This is the last assignment for the tricolor LEDs!  Combine everything that you’ve written so far into one program so you can make everything work from a single prompt! By the way also add white to the colors that you can cycle through in the index.  (white is formed by turning all of the different colors on).

There is no skeleton provided for this assignment.  You should probably design your code similarly to the other assignments though.

09finalProj

This is your last python assignment.  The goal of this project is to design a system that tests the reaction time of the participants.  We are investigating the reaction time of humans to an LED turning on, or their reaction time to and LED turning off.  For now we’ll simulate this by having the program print the words on and off.  Afterwards, we’ll use VBA and excel to analyze some of the data, so we’ll need a way to read it effectively into excel.  For this you’ll be generating a comma separated value(.csv) as output while running the code.  The .csv files from all students will be placed in a single folder and distributed to the class for analysis.

So the first thing to do is to set up the testing functionality.  The test should prompt the user to hit the enter key in response to the light turning on or off.  It should give the user a countdown and then wait 1-4 seconds before activating.  When the enter key is finally pressed the delay time between the light turning on and the key being pressed should be recorded.

Now that the basic rig is created, we should create a function that randomizes the order in which the test is performed.  I recommend keeping track of the number of tests of each kind required and using a random number to pick which one to do, giving each test a fractional proportion as such:

If we need 4 more turn-on tests but only 1 more turn-off test, then we’ll create a random number from 1 to 5 with randint (or multiply random by five).  If the number is less than 5 then it’s an off test, but if it’s 1, 2, 3, or 4 then it’s an on test.  It should work even if there’s only one type left although it might not be the most efficient code in that case.

Now that the tests are randomized, begin creating your csv file.  Open a new file with the title: “output” + your unique identifier.  For example, I might create one called outputFlannelBoy or outputdswarre2 if I wanted to be safe.  Store these data:  The Raspberry Pi that the experiment took place on (a number between 1 and 15), the unique identifier of the particular participant, the type of test performed (on, off), and the time it took for the student to react.

We can finish this assignment up by prompting the user for their unique identifier, and then continuing to prompt for more tests until enough data has been taken.  Now test your system and several other students’!

10VBAnalysis

This is the final assignment.  Now that you have created a small mountain of data along with the rest of your class, it is time to analyze that information.  Rather than do it by hand, which might take some time, we’re going to write a program so if we ever get more data (something that would be likely in an actual job) we’ll have something ready to analyze it and check out the conclusions from it.

Here’s what you need this system to do: