Programming Assignments

Hopefully by now you’ve looked over the tutorial and you know basically how python works.  If you didn’t do this, and don’t know how python works at all, then these assignments might be difficult. But who knows? Maybe you’re born with it. Or maybe it’s Maybelline.

So basically, these assignments will consist of you writing code in python that will be altered to be used in lab.  In lab, we’ll modify our code to work on the raspberry Pi, but for now, our only output and input are the console, so we’ll just pretend that we’re using the Raspberry Pi.

These assignments always start on a fresh page, and will include information about the purpose of the assignment and special information required to complete it.

Once the program is finished, it should react predictably, and you may compare your program to a sample of the desired output.  As long as they act similarly, then you should be fine to move on.

The creation of these programs are useful exercises, but they aren’t the end goal.  Once you arrive at the Raspberry Pi lab, you will be able to convert these exercises so they run and perform specific tasks on your microcontrollers.  This process is detailed at the end of 00example.

Assignment 00Example

This assignment is related to turning an LED on and off.  The way to do that (once it’s hooked up to your circuit) is by toggling the I/O (input output) port that it’s connected to.  So we’re going to pretend that the variable a controls the LED, and that we can turn it on or off by setting it true or false.  This lab is easier with the sleep command from the time library.

The partial code will be in an attached .py file, and will it look something like this:

import time

a = False

# write code to change and print the value of a every second
# in a infinite loop (remember that ctrl-c or command-c
# is the key combination used to escape from an infinite loop.

There will also be an example expected output code that will help you design your code.  For this assignment, it may look something like this, and it’s usually stored in just a pdf.

Python 3.6.0 (v3.6.0:41df79263a11, Dec 23 2016, 07:18:10) [MSC v.1900 32 bit (Intel)] on win32

Type "copyright", "credits" or "license()" for more information.

>>>

= RESTART: C:/Users/David/Documents/TE 205 Python/Assignments/00 Example.py =

True

False

True

False

Traceback (most recent call last):

File "C:/Users/David/Documents/TE 205 Python/Assignments/00 Example.py", line 6, in <module>

time.sleep(1)

KeyboardInterrupt

>>>

Here’s an example solution to the previous problem.  If you had trouble understanding what was expected and/or how to write these programs, don’t worry.  This is the first time these materials have been utilized, and there’s plenty of help available.

import time

a = False

# write code to change and print the value of a every second
# in a while loop that is always true (do you remember that ctrl-c
# breaks you out of an infinite loop)?

while True:

print(a)			#the print lines will have to be changed

sleep(1)

a = not a

In lab we’ll go over how to convert the previous code to the following and run it on a Raspberry Pi, but for now, don’t worry about it.  Just get to the previous step and we’ll be fine

import time

import RPi.GPIO as GPIO

GPIO.setmode(GPIO.BCM) # determines how we refer to pins

GPIO.setup(3, GPIO.OUT) # sets pin 3 to output

a = False

while True:

GPIO.output(3, a) 	# turns on pin 3 if a is true

sleep(1)			#delays for one second

a = not a			#changes the value of a to its negative

Assignment 01OnOff

We’ve learned how to turn a variable on and off on repeat, but it’s time to learn how to control it by keyboard input.  In this assignment, your goal is to set the variable A to a specific setting by keyboard prompts.  If the user types on, it should turn set a to true and print it (turn the LED on), while if they type off, it should do the opposite.

You should have recognized from the previous lines that we’re going to be using input() quite a bit to prompt the user for the next action quite often.  This prompt is going to be interpreted after it’s submitted, and then the status of the LED should be printed.

The skeleton file for this is 01OnOff.py and a sample output can be found called expected.

Assignment 02fade

So you’ve seen how to turn on and off an LED, now it’s time to learn how to control the exact brightness of it.  As you will learn in the lab, LEDs only have two settings (on/off).  As such, it’s quite difficult to get them to truly dim.  But if they flicker quickly, then they may appear dim or bright to the human eye.  We’ll use something called Pulse Width Modulation (PWM) to accomplish this task.  PWM is accomplished by use of a class that controls a few properties of the PWM like frequency, pin, and duty cycle.  I wrote a class to imitate the one you’ll be using on the actual Raspberry Pi, it’s at the top of the skeleton, and it’s big and gross and disgusting, but avoid modifying it anyways or your code might not convert particularly well.

To make an LED dim or brighten, we need to set the duty cycle; however, to some degree we want to learn how to automate this task.  I want to be able to control this LED from the keyboard so I can set it to a specific brightness, and I want to be able to set it to fade to dim, or shine to bright with the keywords fade or shine.  So there are three commands: numbers change the setting on the LED to a different duty cycle, fade makes the LED dim to off, and shine makes it brighten until it’s at full PWM (there’s also an exit command that makes it quit the program!).

The skeleton code for this project is contained in 02fade.py.  It also imports a virtual PWM class that I created for you to practice with, so virtual must be in the same folder as your code when you run it.  Whenever you use the commands associated with virtual, it will act (hopefully) as explained in the beginning of the given code.  Example output is contained in the word file expected.

Assignment 03pulse

At this point, you’ve gone over analyzing input and controlling the PWM leds, so let’s go ahead and write one more function, pulse, before combining them.  Pulse will continuously brighten and dim the LED, so the only input from the user should be the command pulse, and then your program should be fine to run.

You should know everything that you need to for this assignment from the previous, assignments, so feel free to reference them as you make this one.  The skeleton for this code (very skimpy this time) is 03pulse.py.  Like the last assignment it relies on the code in the python file virtual to imitate a PWM, so you’ll need the virtual.py in the same folder when it is run.  The expected output is put in a word doc as per usual, and DON’T FORGET THAT CTRL-C (or cmd-c on macs) IS USED TO BREAK LOOPS IN PYTHON!!!!

Assignment 04LED

Okay guys, you’ve gone and finished all of the previous assignments, now it’s time to combine them into one big granddaddy of an assignment!  In this assignment, we’re going to create a program that can accurately change it’s duty cycle, and can blink and fade and shine and pulse and all of those commands.  This would be a remarkably large main assignment, so it might make sense to pass certain parameters to smaller functions that do most of the actual work.

So I’ve included the example already written into the assignment, and you guys should be able to write the rest on your own because you’re all pretty dang smart (sorry but we have high expectations once you make it to college).  Still if you have any questions, feel free to ask Dr. Jasper or myself for help!

The skeleton file is 04LED.py, and once again, it relies on the virtual.py, so don’t run it if virtual.py isn’t in the same folder.  An expected output isn’t included for this assignment, because the expectations are the same as for the other assignments (and because I didn’t want to make one). Good luck!

and DON’T FORGET THAT CTRL-C (or cmd-c on macs) IS USED TO BREAK LOOPS IN PYTHON!!!!

Assignment 05 TriColor Control

Okay, that last assignment was about as gross as they get.  This next one is much much simpler.  It still has the nasty class at the top because we need a virtual class to practice on, but this code will read more smoothly hopefully

So the main goal of this code, is to activate a Tricolor Led, which has three different LEDs inside of it.  So the big difference (which isn’t really much of a difference) is controlling all those different colors at once.

Objectives are the LED must initially turn on with only one color.  If a button is pressed (interpreted as a 1 input on the keyboard) it should cycle to a different color, and if you type a specific color it should turn the rest off and that one on at duty cycle 75, and if you type a 9 digit color code it should make that exact color (redgreenblue).  For example: 100000050 would set red to 100, and green to 000 and blue to 100.

There is a skeleton again, and an expected output that prints what the interaction between the program and the user should look like.

Assignment 03. Rainbow!

I think that putting exclamation signs into filenames is bad juju, but we’ll see how this works out.  Anyways, the goal of this one is to make your tricolor LED slowly pulse in a rainbow!  This happens when each color reaches a peak, one at a time, and at least two colors are on all the time.  It’s also kinda pretty which is why I like it.

So the objective is to write a code that increases one color’s duty cycle to 100 and the decreases it again afterwards to zero, but while decreasing, another color should be increasing.  This just keeps looping, and if you did everything right you should get the same output as before

There is a skeleton again, and an expected output that prints what the interaction between the program and the user should look like.